AMR Fleet Management — Combined Literature Review Table

| Article Title | Max AMRs | Sim / Real | Map / Graph Type | Environment | Control Architecture | Planning Approach | Methods |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A Multi-Agent System Using Fuzzy Logic to Increase AGV Fleet Performance in Warehouses [45] | 11 | Only simulation | Roadmap graph | Warehouse | Decentralized | Hybrid-adaptive (fuzzy logic + A*) | Fuzzy logic, A*, Fuzzy Inference system for priority |
| A Routing and Task-Allocation Algorithm for Robotic Groups in Warehouse Environments [41] | 6 | Only simulation | Grid | Warehouse | Centralized | Mostly deterministic (Dijkstra + Kuhn-Munkres, some adaptive) | Dijkstra, Kuhn-Munkers algorithm |
| AGV Traffic Management in Automated Industrial Plants: An Enhanced Lifelong Multi-Agent Path Finding Approach [22] | 5 | Only simulation | Multi-layer (gridmap + roadmap + topological) | Industrial | Centralized | Hybrid (Lifelong MAPF/CBS deterministic + continuous replanning) | Lifelong Multi-Agent Path Finding, Rolling-Horizon Collision Resolution, Conflict-Based Search |
| An Integrated Timetable Optimization and Automatic Guided Vehicle Dispatching Method in Smart Manufacturing [40] | 8 | Only simulation | Prescribed guide paths | Manufacturing | Centralized | Hybrid (genetic algorithm adaptive + deterministic timetable) | Timetable optimization and AGV dispatching, Genetic algorithm |
| Anytime Lifelong Multi-Agent Pathfinding in Topological Maps [51] | 140 | Only simulation | Topological graphs | Warehouse | Centralized | Hybrid (ECBS/Corridor-CBS deterministic + dynamic time horizon) | Lifelong Multi-Agent Path Finding, Rolling-Horizon Collision Resolution, Safe Interval Path Planning, Corridor-Conflict-Based search |
| Autonomous Transport Vehicles: Where We Are and What Is Missing [27] | 5 | Both simulation and real | Trajectory envelopes + state lattice | Industrial | Centralized | Adaptive | Trajectory Envelopes, Lattice-based motion planning, Constraint-based control |
| Centralized versus Distributed Non-linear Model Predictive Control for Online Robot Fleet Trajectory Planning [26] | 10 | Only simulation | Continuous space | Not specified | Both centralized and decentralized (compared) | Adaptive (NMPC deterministic formulation, adaptive planning) | Non-linear Model Predictive Control |
| Collision-free allocation of temporally constrained tasks in multi-robot systems [42] | 150 | Only simulation | Grid + undirected weighted graph | Generic | Centralized (CFAT-D, auction paradigm) | Deterministic (SIPP path planning) | Collision-free Allocation of Tasks having Deadlines, Auction-based allocation, Safe Interval Path Planning |
| Conflict-Based Task and Motion Planning for Multi-Robot, Multi-Goal Problems [52] | 4 | Only simulation | Roadmap | Generic (warehouse/manufacturing) | Centralized | Hybrid (temporal planner deterministic + adaptive integration) | Probabilistic Roadmap, Domain Description Language-based task planner with OPTIC planner, Dijkstra |
| Deadlock avoidance algorithm for AGVs on a tessellated layout [50] | 8 | Only simulation | Tessellated layout (grid/occupancy tiles) | Industrial | Centralized | Deterministic (tessellated layout) | Tessellated environment, reservation-based priority |
| Decentralized and prioritized algorithm for AGV fleet management [24] | 8 | Only simulation | Grid/mesh | Generic | Hybrid | Deterministic (reclassified) | Dijkstra, Prioritization algorithm based on node degree and state, Optimization subroutines |
| Decentralized Market-Based Task Allocation Algorithm for a Fleet of Industrial Mobile Robots [43] | 6 | Only simulation | Follow magnetic line | Factory | Decentralized | Hybrid (market-based auctions, mostly deterministic) | Sequential, decentralized market-based algortihms |
| Development of a Fleet Management System for Multiple Robots’ Task Allocation Using Deep Reinforcement Learning [47] | 3 | Only simulation | Grid | Generic | Mostly centralized (some decentralized path planning) | Adaptive (DRL) | Deep Q-network |
| Distributed Fleet Management in Noisy Environments via Model-Predictive Control [33] | 20 | Only simulation | Stochastic Work Graph (roadmap with vertices/edges) | Industrial | Decentralized | Adaptive (replanning, stochastic, learning-based) | Two-level path planning, Stochastic Work Graphs, MPC solving with Q-learning |
| A dynamic routing strategy for the traffic control of AGVs in automatic warehouses [18] | 3 | Real tests | Roadmap graph | Industry | Centralized | Deterministic (close to adaptive) | A*, Traffic Estimation, Dynamic Replanning |
| Efficient Multi-Robot Cooperative Transportation Scheduling System [37] | 4 | Both simulation and real | Not specified | Factory | Centralized (master-slave) | Deterministic (priority-based, fixed index) | Master-slave architecture, communication standard, Priority waiting |
| Efficient multi-robot path planning in real environments: a centralized coordination system [29] | 2 | Both simulation and real | Graph | Industrial | Centralized | Deterministic (TEA*, with replanning) | TEA*, Graph-based, Vertices blocking mechanism, Trajectory supervision |
| An End-to-End Deep Reinforcement Learning Based Modular Task Allocation Framework for Autonomous Mobile Systems [46] | 5 | Both simulation and real | Occupancy grid + continuous workspace with waypoints | Warehouse inspection | End-to-end DRL | Adaptive | Dijkstra, Dynamic Window Approach, Deep Reinforcement Learning, Multiple Travelling Salesmen Problem. |
| Ensemble Coordination Approach in Multi-AGV Systems Applied to Industrial Warehouses [23] | 30 | Only simulation | Multi-layer (roadmap + topological) | Industrial warehouse | Hierarchical (centralized + decentralized) | Adaptive (D*/MPC high-level; A* low-level) | Local Negotiation, Roadmap generation, Decentralized coordination |
| Fleet Management System for Autonomous Mobile Robots in Secure Shop-floor Environments [35] | 3 | Real tests | Zone/region with topological elements | Shopfloor | Centralized | Mostly deterministic (pre-made sub-plans) | Hierarchical task decomposition, multi-criteria task allocation, pre-made sub-plans, Restricted zones |
| Fleet Management System for an Industry Environment [34] | 10 | Only simulation | Graph | Generic | Hybrid | Deterministic | A*, One-way and bidirectional paths, Semaphore-based priority |
| Hierarchical and Flexible Traffic Management of Multi-AGV Systems Applied to Industrial Environments [20] | 8 | Both simulation and real | Three-layers (roadmap + topological + topological based) | Industrial | Hierarchical (hybrid) | Hybrid (Dijkstra deterministic + traffic-sensitive adaptive) | Three-layer control architecture, Dijkstra |
| A hierarchical approach for joint task allocation and path planning [39] | 90 | Only simulation | Undirected graph + grid | Automated warehouse | Hierarchical (hybrid) | Deterministic (k-means + A*, offline conflict-free planning) | Three-level task allocation and path planning solver, Low-level A*, High-level Conflict-Based Search |
| Hierarchical Traffic Management of Multi-AGV Systems With Deadlock Prevention Applied to Industrial Environments [21] | 8 | Both simulation and real | Three-layers (roadmap + topological + topological based) | Industrial | Hierarchical (hybrid) | Adaptive (dynamic replanning, traffic-aware) + Deterministic (Dijkstra) | Dijkstra, Time-expanded graphs |
| Highly-scalable traffic management of autonomous industrial transportation systems [25] | 50 | Both simulation and real | Two-layer (topological + state-lattice) | Industrial | Decentralized | Hybrid (state-lattice deterministic + local negotiation adaptive) | State-lattice representation, lattice-based path planning, priority-based negotiation, private-zone mechanism |
| Integrated Motion Planning and Coordination for Industrial Vehicles [28] | 10 | Both simulation and real | Lattice-based, grid | Industrial | Centralized | Adaptive (least commitment, trajectory envelopes, online refinement) | Lattice-based path planning, Trajectory envelopes |
| Layered-Cost-Map-Based Traffic Management for Multiple AMRs via a DDS [16] | 2 | Real tests | Layered costmaps with cells | Generic | Centralized | Deterministic | Layered Costmap |
| Mitigating Emergency Stop Collisions in AGV Fleets in Case of Control Failure [53] | 45 | Only simulation | Continuous (trajectory planning) | Intralogistics | Centralized | Deterministic | Mitigate collisions in emergency stop scenatios, delay, optimization and replanning methods |
| Multi-Robot Task Allocation and Scheduling Considering Cooperative Tasks and Precedence Constraints [49] | 3 | Only simulation | Directed path graph | Generic | Centralized | Deterministic | Feasibility criterion, greedy constructive heuristic and local improvement heuristic |
| Multiple Robots Avoid Humans To Get the Jobs Done: An Approach to Human-aware Task Allocation [44] | 18 | Only simulation | Grid | Shopping centre / small warehouse | Decentralized | Adaptive (human-aware, dynamic env models, auction-based) |  |
| Multi-robot cooperation in the MARTHA project [30] | 10 | Both simulation and real | Topological graph | Harbors, airports, marshaling yards | Decentralized | Adaptive | Plan-Merging Paradigm, Spatial resource allocation |
| Online Conflict-Free Scheduling of Fleets of Autonomous Mobile Robots [36] | 750 | Only simulation | Weighted, undirected, connected graph | Warehouse | Centralized | Deterministic | Dijkstra, Spatiotemporal conflict avoidance, Tailor-made scheduler |
| Operating a large fleet of mobile robots using the plan-merging paradigm [31] | 30 | Both simulation and real | Topological graph | Industrial | Decentralized | Adaptive (local, incremental, distributed) | Plan-Merging Operation |
| Path Planning and scheduling for a fleet of autonomous vehicles [48] | 4 | Only simulation | Continuous, B-Spline surface | Industrial | Centralized | Mostly adaptive (with deterministic structure) | Bump-Surface environment representation, Genetic algorithm |
| Predicting Conflict Zones on Terrestrial Routes of Automated Guided Vehicles with Fuzzy Querying on Apache Kafka [54] | 2 | Only simulation | Predefined routes + roadmap | Smart factory | Centralized | Adaptive | Fuzzy logic, Fuzzy real-time data stream processing |
| A Probabilistic Eulerian Traffic Model for the Coordination of Multiple AGVs in Automatic Warehouses [19] | 15 | Only simulation | Multi-layer (roadmap + graph cells) | Warehouse | Hierarchical (hybrid) | Adaptive (probabilistic, MPC, replanning, TEN) | Eulerian Traffic model, Time-expanded Network |
| Queue Formation and Obstacle Avoidance Navigation Strategy for Multi-Robot Systems Based on Deep Reinforcement Learning [55] | 20 | Only simulation | Continuous environment | Not specified | Decentralized | Adaptive (DRL) | Queue formation, Deep reinforcement learning, Deep Deterministic Policy Gradient |
| A rules and communication based multiple robots transportation system [32] | 3 | Only simulation | Two-level (topological + trajectory) | Generic structured env. | Decentralized | Deterministic | Two-level path planning (topology and trajectory), Rule-based traffic control |
| Smart mobile robot fleet management based on hierarchical multi-agent deep Q network towards intelligent manufacturing [38] | 4 | Only simulation | Continuous space | Manufacturing | Hierarchical (hybrid) | Adaptive | Hierarchical Reinforcement Learning Algorithm, Multi-agent reinforcement learning algorithm |
| Traffic Management of Multi-AGV Systems by Improved Dynamic Resource Reservation [17] | 10 | Only simulation | Manhattan grid-like, weighted, undirected graph | Industrial | Centralized | Mostly deterministic (shortest path + real-time conflict resolution) | Reservation-based, IDRR |